Computers 4 iDiots

Tutorial Four: Create Logic Gate models

| Activity | Description | Time |
| --- | --- | --- |
| Introduction | The previous tutorial established the link between binary numbers and electronic logic circuits. These circuit models showed the switches, which affect the output from the circuit, to clearly demonstrate the idea of an input being ON or OFF in order to achieve a particular output. This tutorial takes this one stage further, by introducing the key idea of Logic gate symbols, which can be used to design computer systems in order to achieve outputs according to the needs of a variety of logic (truth) tables. Establish the links indidcated above, then introduce the main learning aims of this tutorial, outlined on slide 1, as follows: - To find out what Logic gates are, and how they are used to design computer systems - To find out what the seven different types of Logic gate are, and how they can be used with logic (truth) tables to create systems with particular outputs - To develop the electronic circuit models into Logic gate models - To create the model of a simple binary adder, by combining two Logic gate models In order to achieve the final two aims, it is important that students have completed all four of the models in the previous tutorial. This is because these models (and their code) can be readily adapted to produce the seven logic gate models. If this code is not available, (for example if a student has been absent through illness) it is worthwhile either asking the student to complete the previous tutorial first, or if this time is not available, supplying the code to the student (for example, from a model you have produced yourself when familiarisation with these materials). | 10 minutes |
| Key learning: Logic gates | Use slides 2 through to 5 to introduce the seven different types of logic gates. Slide 2 introduces the seven symbols, and names them. Discuss what each of the symbols might represent, in terms of its likely output. Three of the symbols (AND, OR, NOT) relate directly to the electronic logic models created previously. Reinforce the idea that these symbols perform exactly the same function, but that the switches built into each logic gate are not explicitly shown. However, they are still there, and operate in exactly the same way. Slide 3 describes the AND, OR, and NOT logic gate symbols in more detail. Draw attention to the way the output from the logic gates is shown in the logic (truth) tables and to the use of Q to represent the output, to avoid possible confusion with a capital O, and a (binary) 0. Finally, point out the use of the circle in the NOT symbol, to indicate that the output has been inverted, i.e. is opposite from the single input in this gate. This circle reappears in several other gate symbols shortly. Slide 4 describe the NAND and NOR logic gate symbols. Explain that both of these symbols are the equivalent of combining two circuits. The NAND symbol combines the AND symbol with the NOT symbol, and the NOR symbol combines the OR symbol with the NOT symbol. Compare the logic table outputs for both the AND/ NAND and OR/NOR symbols by returning to the previous slide. Indicate that the NAND / NOR gates are also called universal gates because they can be used to create complex systems when combined into various configurations. Slide 5 introduces exclusive OR and exclusive NOR gates. Begin by focusing on the meaning of the word exclusive, and its connection with its root, to exclude. Something has been left out, but what is it? See if students can define this difference by comparing the logic tables with the previous OR / NOR tables. Highlight the differences. Whereas the OR/ NOR gates produce three similar, one different output, the XOR / XNOR gates both produce two similar, two different outputs, useful when designing particular configurations, and used in the binary adder later. | 20 minutes |
| Activity: Create logic gate models: Introduction | Introduce the task of developing the electronic logic circuit models to create the seven logic gate models. (Slide 6) Ask students to download the zipped folder of graphic resources from the Resources link. Ask students to examine the completed models, which are available from the Examples link. In particular, ask them to look at the link between interacting with the model, and with the appearance of its truth table. Indicate that apart from writing new code to control the truth table, which only has to be done once, and can then be used on each model, the main activity is editing both graphics and code from the circuit models by removing unnecessary sections. Code also needs to be combined from more than one circuit model to produce some of the logic gate models. | 10 minutes |
| Activity: Create logic gate models | The demonstration on slide 7 can be used independently by students to produce the models, in the majority of situations. Here are the main tasks required to produce each of the seven logic gate models: AND gate - Save the AND circuit document with a different filename - Delete the battery and terminals layer, as these are not required - Delete the switches layers (A and B) as these are not required - Create a new layer for the logic table movie clip - Edit the circuit movie clip, so that it has only a single output to its bulb - Import the logic gate graphic to the stage and move it into place - Reposition the two input buttons, so that they resemble the logic gate symbol arrangement, at the left hand side of the symbol - Edit both input movie clips, so that they contain a single circuit connecting them to the symbol. Change the colour of the circuit on frame 2, inside the movie clip, to red to show its ON state. (It is not possible to use code to recolour the input symbols, as this would affect the colour of the buttons themselves). - Test the logic gate model. It should already work, as the code remains the same as with the circuit model. - Edit unnecessary code, by using comments to disable them. All the switch colour objects, functions, and references are not needed. (Leaving this code does not create problems, but it helps to clarify the changes which have been made). - Create the logic table movie clip, which requires four frames, one for each of its four possible output states, as follows: - Import the background graphic. - Populate the table with the required input and output values, by entering 0 or 1 into each static text box, as required. - Create a highlight graphic, which has to be moved into place to highlight each row of the truth table, according to the frame it is on. - Write the changeLogicTable() function, which sends the logic table movie clip to a particular frame, according to the state of the input buttons (and their Boolean variables). (Students may be able to do this independently: challenge them to do so). - Call the changeLogicTable() function whenever either of the input button functions is run (i.e. the buttons are clicked). OR Logic gate It is straightforward to adapt the AND gate model to create the OR gate model. These are the required steps: - Save the AND gate model document as or_gate.fla - Change the AND gate graphic to the OR gate graphic - Edit the completeCircuit() function so that the circuit is completed if either of the inputs is ON. This involves changing the AND operator (&&) in the IF statement to the OR operator (¦¦). - Edit the logic table movie clip so that it displays the correct outputs for the OR gate. NOT logic gate The NOT logic gate model requires slightly more editing from the AND gate model, but once again, this is relatively straightforward. These are the required steps - Save the AND gate model document as not_gate.fla - Change the AND gate graphic to the NOT gate graphic - Delete inputB and all references to it in the code - Edit the logic gate movie clip and its code to handle the two states of its single input. (A different background image is required). - Edit the inputA movie clip function so that the circuit and bulb ON/OFF functions are reversed. This is exactly the same procedure as was required to edit the NOT circuit model. NAND and NOR gates - The AND and OR gate models can quickly be changed to NAND and NOR models, by reversing the output of the AND and OR gates. The required steps are: - Save the AND gate model as nand_gate.fla, and the OR gate model as nor_gate.fla - Change the gate graphics in both models - Edit the logic tables to show the correct outputs - Call the circuitOn() and bulbOn() functions as soon as the models load, so that both are in their ON state initially. - Edit the completeCircuit() function in both models so that the circuit and bulb ON/OFF functions are reversed (as has just been done for the NOT gate model The XOR gate The Exclusive OR model requires a little more coding, as follows: - Save the OR gate model as xor_gate.fla - Change the gate graphic - Edit the logic table movie clip to show the correct output - Edit the completeCircuit() function so that it has IF, ELSE IF and ELSE statements. The IF statement, and the ELSE if statement handle the states when the inputs are different. The ELSE statement handles the two states when they are the same. The XNOR gate The XNOR gate produces the opposite output to the XOR gate, so the required steps are as follows: - Save the NOR gate nodel as xnor_gate.fla - Change the gate graphic - Edit the logic table movie clip to show the correct output - Copy and paste the completeCircuit() function from the XOR model (or rewrite the function). - Reverse the circuit / bulb ON/OFF functions in the completeCircuit() function to produce the opposite outputs. | 60 minutes |
| Activity: Create a binary adder model | The final model is of a simple binary adder, which adds two binary digits together, producing an output, and a carry. You may like to consider this as an extension task for the more able students in the group (depending upon the group of course), but it is recommended that all students should examine the example version of the model (from the Example link) in order to see how electronic circuits can be used to perform binary calculations. Explain that the binary adder is created by combining two logic gates together to produce the required output / carry results, depending upon the state of the two inputs, which correspond to the two binary digits being added. Check that students have located the background graphic (simple_adder_background.png) which is required for this model, in the zipped folder they downloaded previously. (Slide 8) Examine the truth table at the beginning of the demonstration (slide 9), which shows the output / carry results. Ask students to identify which of the logic gate models they have just created produce these required results. Discuss how the models could be arranged so that both gates use the same two inputs. (Check with the the completed example to confirm how this works). Discuss how the XOR logic gate can be used to handle the output, and the AND logic gate can be used to handle the carry. The steps to create the model are then as follows. First, the components need to be edited or changed: - Save the XOR model as simple_adder.fla - Change the size of the document to accommodate the truth table to the side of the model. - Import the AND gate graphic and position it below the XOR graphic - Edit the Input A movie clip so that the circuit connects to the AND logic gate, as well as the XOR gate. - Repeat this process for the Input B movie clip - Copy the bulb movie clip, and move it into place to form the Carry from the AND gate. Label the Output Q, and the Carry C by placing a text box on top of each of the bulb movie clips. - Rename the components as required, using instance names which refer to either the XOR or the AND gate. - Edit the logic table movie clip, so that it has both the results of the Output and the Carry by importing the background graphic, and editing the highlight graphic. The code then has to be edited and combined from the XOR and AND models. All references to the circuit and bulb in the original model have to be edited, so that they have the xor_ prefix. The references have to be edited as follows: - The bulb and circuit colour objects have to be renamed. - The circuitOn() and circuitOff() functions have to be renamed. - The bulbOn() and bulbOff() functions have to be renamed. - The completeCircuit() function has to be renamed. It then has to be edited to call the renamed functions, listed above. - The Input A function has to call the newly renamed xor_completeCircuit() function. - The Input B function has to be edited to call the newly renamed xor_completeCircuit() function. The code from the AND gate model has to be edited before it is inserted into adder model, as follows: - Copy all the code from the original AND gate model into a text editor such as Notepad. - Remove all references to the inputA and inputB buttons, and to the logic table, as these already exist in the adder model, and must not be duplicated. - Delete the two Boolean variables - Delete the inputA ON / OFF functions, and the inputB ON / OFF functions - Delete the inputA_mc function, and the inputB_mc function - Delete the changeLogicTable() function - Edit the remaining code so that the circuit, bulb and completeCircuit() functions have the and_ prefix. - Insert this edited code into the adder model - Edit the inputA and inputB functions to call the newly named and_completeCircuit() function. | 40 minutes |
| Activity: Further research | The research task (slide10) can be used as an extension task for students who have completed the models ahead of the rest, or as a homework task. | 30 minutes |
| Review Key points | Use the questions on the final slide in the tutorial to emphasise the key learning points in the tutorial. Choose questions according to how far the class has progressed during each lesson in the time available. | 10 minutes |
| Resources | - Computers 4 iDiots tutorial 4 - Logic gate examples (from the Examples link) - Logic gate models graphics (zipped folder from the Resources link) |  |