Computers 4 iDiots

Tutorial Three: Create electronic circuit models

| Activity | Description | Time |
| --- | --- | --- |
| Introduction | This tutorial links the previous learning about binary numbers to the use of electronic circuits, in order to store and process data on a computer system. Outline the four main aims of the tutorial, as indicated on slide 1: - To show how a computer is a complex arrangement of electronic switches, which can be used to store binary digits. - To create the model of a simple circuit, which demonstrates the use of an input to control a switch, and so turn the output ON or OFF. - To develop the simple circuit model to create three further electronic logic circuits (AND, OR, NOT). - To show how logic circuits can be used to produce different outputs, depending upon whether inputs are ON or OFF, and to identify these outputs in a truth table. | 10 minutes |
| Key Learning: How computers use switches | Emphasise the key idea behind the use of electronic switches to handle binary numbers, using slide 2 as a starting point. Refer back to the number system developed in tutorial one, using fingers to represent digits. Revise how each finger can be in an ON or OFF state, in order to represent a different value. Revise how the binary system has only two digits, 0 and 1, and indicate that these can be represented as an OFF or ON state, which makes a electronic swith an ideal method of handling them. | 5 minutes |
| Information: Electronic switches | Run through the information provided about the history of electronic switches on slides 3 and 4. Discuss how the diode valve can act as an electronic switch, by allowing current to flow when a charge is applied to the cathode, causing it to heat up. Indicate the disadvantages of these types of valves, namely the heat generated, the size of the valves, and the likelihood of them failing. Compare diode valves to transistors, which perform a similar function, but without the disadvantages of heat, and size. Finally, link this to integrated circuits, which engineer millions of tiny transistors on to a small silicon chip. | 10 minutes |
| Activity: Create a simple circuit model: Introduction | Introduce the first activity for this tutorial, which is to produce a simple circuit model, containing a single switch, which turns a light bulb ON or OFF, to demonstrate its output. Examine the simple circuit model by looking at the example (follow the Example link). (Slide 5). Run through the main components required in the model, namely the circuit, an input, a switch, and a bulb. Explain that the first stage in producing the model is to create the graphics required by using the drawing tools in Flash. Indicate that these graphics can be re-used in the other models to be developed shortly. | 5 minutes |
| Activity: Create a simple circuit model: Create the graphics | The demonstration (slide 6) shows students how to create the graphics required for the simple circuit model. Student can run through this demonstration independently, as there are no significant difficulties, providing they are familiar with the Flash environment and its tools. Indicate that the graphics have to be created in Flash, as it is necessary to convert them to symbols, so that their colour can be changed by using code. The main stages are as follows: - Create the circuit movie clip symbol by using the rectangle tool, and then erasing sections for the battery, and the switch. - Create the battery and terminals graphic symbols. - Create the bulb movie clip symbol. - Create the switch movie clip symbol, using the Free transform tool to rotate it into its open / closed positions on two frames in the clip. - Create input button symbols A and B, using the graphics provided from the Resources link. | 20 minutes |
| Activity: Create a simple circuit model: Write the code | Provide an overview of the requirements of the code for the model, using the information on slide 7. Some new Actionscript learning may be involved here, using colour objects to change the colour of several of the movie clip symbols. As this uses hex numbers to identify colours, the demonstration also includes some key learning about the hex number system. It may be useful to cover these ideas with the whole class during the demonstration. Ask students to download the Code guide to assist in creating the models (or provide printed copies to use “off screen”). The stages involved in writing the code are as follows: - Give each of the components a suitable instance name, as indicated in the demonstration (and in the Code Guide). - Create colour objects for each of the symbols which changes colour (circuit, bulb, switchA). - Write pairs of functions for each of the components which changes colour, one function for the ON state, the other for the OFF state. - Test and troubleshoot the circuitOn() function. Controlling the circuit The flow diagram at this point in the demonstration shows how a Boolean variable is used to check the state of the input. The input button function is used to toggle between the ON / OFF states. Some students may be able to write this function independently, and are invited to do so. - Create the inputA button function, to turn the circuit, switch, and bulb ON, when it is clicked, changing the state of the Boolean variable on each click so that the components can be turned ON and OFF by calling the appropriate pairs of functions. | 30 minutes |
| Key learning: Electronic logic circuits | Explain how electronic circuits can be used to create the same outputs as some logic (truth) tables. Use slide 9 to examine the other three models (AND, OR, NOT) and to establish the reason for their logical names. (“The AND circuit needs both inputA AND inputB to be ON in order for the bulb to light.” Provide similar explanation for the OR and NOT circuits). Compare the output from the circuits with the logic table example (slide 10). This table shows the possible inputs / outputs for two binary digits. Draw attention to the Carry column, where the output is exactly the same as the AND circuit. Indicate that the AND circuit might therefore be used to generate this output in a computer system. Other circuit systems may be applicable to other logic table outputs. | 10 minutes |
| Activity: Create the AND, OR, and NOT circuits | The other three circuit models can be made by editing and developing the simple circuit model. This requires making changes to the graphic components, and by editing / writing new code. Students may be able to follow the demonstration (slide 11) independently to produce these models. The tasks may be outlined as follows: AND circuit - Insert a second button, InputB, and give it a suitable instance name. - Edit the circuit movie clip so that it has a second gap for another switch, in series with the first. - Duplicate the terminal graphics. - Create a second switch from the Library; give it a suitable instance name. - Create a new colour object to change the colour of switchB. - Create a second Boolean variable to test the state of inputB. The IF statement which tests the state of these two variables, requires the use of the && (AND) operator. Explain this to the whole class if they have not met this operator before. - Duplicate functions to turn swithcB and inputB ON and OFF. - Write a new function which completes the circuit when both inputs are ON. - Edit the inputA function so that it only operates switchA. - Duplicate and edit the inputA function for inputB. - Test and troubleshoot the AND circuit The OR circuit Examine the OR logic table to highlight the differences between this and the AND logic table. Discuss how this third model can use the majority of the AND model code, but the logic of the completeCircuit() function needs to be changed. The changes required are as follows: - Save the AND circuit model as OR circuit. - Edit the circuit movie clip so that the switches can be arranged in parallel instead of in series. - Move the terminals and switches to arrange them in parallel. - Change the IF statement in the completeCircuit() function from using the AND operator, to using the OR operator. (This is the only change needed to the code). The NOT circuit The NOT circuit inverts the output of the simple circuit, and its logic may cause some initial confusion. The example of the emergency button to stop a machine helps to demonstrate its purpose. The simple circuit has to be edited as follows: - Save the simple circuit model as the NOT model - Edit the rotation of the switch to indicate that when it is turned on, it breaks, rather than makes, the circuit. - Call the circuitOn() and switchOn() functions as soon as the model loads, so that the circuit is initialised in its ON state. - Change the calls to the circuitOff() / circuitOn() and bulbOff() / bulbOn() functions in the inputA button function, so that these components are in their opposite states when the button is clicked. | 45 minutes |
| Review Key points | Use the questions on the final slide in the tutorial to emphasise the key learning points in the tutorial. Choose questions according to how far the class has progressed during each lesson in the time available. | 10 minutes |
| Resources | - Computers 4 iDiots tutorial 3 - The Code guide document (from the Resources link). - The input buttons graphics (from the Resources link). - The four circuit models (from the Examples link). |  |