Computers 4 iDiots

Tutorial Five: How the CPU works

| Activity | Description | Time |
| --- | --- | --- |
| Introduction | Introduce the main aims of this tutorial, which are as follows: - To find out what the Central Processing Unit (CPU) in a computing device is, and how it connects to the other main components in a computer system. - To find out what the main parts of a CPU are, and the functions these perform. - To find out what a program cycle is, and how this consists of four main stages; fetch, decode, execute, and store. - To create a CPU model in Flash, which demonstrates the task of adding two numbers together, and comparing the resutl with an answer input by a user. | 5 minutes |
| Information: What is the CPU? What the CPU model does | Discuss how the CPU of a computing device is often described as the “brain” of the system. Indicate that while it can perform some similar functions to the human brain, like calculations, and often much more quickly, it is not able to “think” in the same way that a human can. (Though computer scientists strive to replicate this process with ever more complex systems). Another analogy might be to the human heart, with which the CPU also shares similarities. The heart pumps blood around the human “system”, while the CPU moves electronic messages and data around a computer system, controlled by the continuous beat of its internal clock. Explain the purpose of the CPU model, so that students understand what it demonstrates before examining the model. It is based on a simple type of mathematics program, which displays two numbers on a screen, and asks the user to input the answer to their addition. Emphasise that the model demonstrates two main features of the CPU, performing calculations, and performing logical operations. Give some examples of logical operations (Equal to, Greater than, Less than) to clarify what a logical operation is. In the case of the model, the CPU compares whether the user’s answer is equal to the correct answer calculated by the CPU. | 5 minutes |
| Key learning: How does the CPU work? | Discuss how it is necessary to understand what the main parts of a computer system are, and how the CPU connects to these, before beginning work on creating the model. Similarly, it is necessary to understand the functions of the parts of the CPU in order to see how data is passed from one part of the system to another, and is processed, either by performing calculations, or making comparisons. Stress that an actual CPU in a modern computing device is more complex than the model being examined, but that the basic way the model works illuminates the way that instructions are carried out upon data stored in memory, or input by a user. Ask students to take a look at the example CPU model (available from the Example link) before running through the demonstration (slide 4) which explains how the CPU connects to and works with the components in the system. Advise them that they are unlikely to fully understand the model until they know more about its operation. Collect and list questions about the model which are puzzling students after first examining the model (e.g. What is RAM? What is a register? etc.) Tick off the answers to these questions as you run through the explanation which follows: - All computing devices, not just PCs, have CPUs. - A system has input and output devices, so that users can interact with the device, and get output, after information has been processed by the system. - There are many different types of input and output devices, including those like a touch screen, which integrates an input and output device. - The motherboard consists of a large number of printed circuits, which connect all the components of the system together. Discuss the advantages of using printed circuits instead of wires, and how components fit into a motherboard. Introduce the key idea of a bus, which is used to describe how data is moved from one component to another. - The BIOS is a memory chip which stores vital information about the system, so that the computer can connect the components in its system. Use the comparison of memory loss to explain the consequences of a BIOS failure. Check that students understand BIOS refers to Basic Input Output System, and that it has to be stored in a type of memory which cannot be changed by the user, and so is Read Only Memory (ROM). - RAM stands for Random Access Memory, which can be accessed at any time by the user, or by a program. Instructions from programs are stored in RAM when the program starts. Documents are stored in RAM, until they are saved. RAM is cleared when the computer is switched off, which explains how work can be lost if a system “crashes”. - The Hard Disc Drive is used to store programs when they are installed on a machine, or to store work which has been saved by the user. Discuss how the drive consists of spinning platters, and a head, which reads magnetic data from the drive as it spins. - The CPU sits in the middle of all these other components, carrying out the instructions of programs, and connecting the components by means of buses. The CPU consists of millions of transistors, creating logic gates and adders to process data. - The fetch, decode, execute, store cycle describes how the CPU fetches an instruction or data from RAM, decodes the instruction so that it can be processed electronically, carries out the instruction, and stores the result (if necessary). This cycle is fundamental to any computer processing system. - The CPU model has three main sections; RAM, the CPU control unit, and the execution unit. The control unit organises how a task has to be carried out (fetch, decode). The execution unit consists of the arithmetic and logical unit, and registers. The parts of the system perform the following functions: - RAM has many millions of memory cells, each with its own address, like a Post Code, so that the CPU can find instructions or data. - The Program Counter loads addresses in sequence and increments to the next address after each program cycle. - RAM stores instructions, data, or even other addresses. Instructions are written in “assembly language” which people can understand. - The address bus sends addresses to and from RAM, the data bus collects and sends data to RAM, and the control bus sends signals to enable the other components. - Registers are temporary memory stores inside the CPU. - The instruction decoder converts an instruction in assembly language into machine language, binary code which can be processed by the electronic components. - The signal generator sends signals which enable other components, so that they can perform tasks. - The ALU can perform arithmetic calculations, or logical operations (such as comparing data). The result of a logical operation is stored in the Flags register (either TRUE or FALSE). - A program cycle occurs when the fetch, decode, execute, store process for a single instruction has been completed. - The CPU has an internal clock (actually a vibrating crystal) which controls how everything happens in the correct order. This is the equivalent of the Next button in the model. After discussing the key learning, ask students to take another look at the CPU model, and see how it works in the light of their new understanding. Ask them to list any questions which are still puzzling them, and refer back to the demonstration to answer these as necessary. | 45 minutes |
| Activity: Set up the CPU model | Introduce the process of creating the CPU model, emphasising that the main difficulty lies not in the complexity of the code, but in the number of components, and small functions required to make it work. Ask students to download the resources required to produce the model, which are in a zipped folder (from the Resources link). The folder contains the graphics needed to produce the model, and a CPU Code Guide, which has the messages used in the model, and a guide to the functions needed for each program cycle in the model. It is useful to provide students with both an electronic and printed version of the code guide. The electronic version is used to copy and paste, the messages, while the printed version is easier to use “off screen”. Look through either the printed or electronic version of the guide with students, so they can see how the program cycles are laid out, and how to use the guide. - Run through the list of components on the first slide of the demonstration (slide 7) so that students understand the different types of component, and what they do. - Check that students understand how the printed circuits on the motherboard form “buses”, and that they have to draw lines on top of these buses to show they are active, when producing many of the movie clips shortly. - The text boxes are used to display messages, or addresses, instructions, and data, as the model develops. Check that students know to use dynamic text boxes, and that each box will require a unique instance name, ending with the _txt suffix. - Creating the RAM component is fairly straightforward, but check students understand why 9 frames are needed, even though there are only 8 address cells. The first frame in the movie clip is used when no cells in RAM are active. Each cell will therefore be on a frame which is one more than the cell number. For example, cell 1 exists on frame 2, and so on. The highlight used to show an active cell uses the same process as that used with the truth table in previous models. Make this link, so that students can see how it works. - Emphasise the value of checking each component as it is created, using the methods shown in the demonstration. Knowing that the component is “good” makes possible troubleshooting easier later in development. - The registers are simple components, which should create little difficulty, providing that students are aware of how to access the dynamic text box within each register movie clip. Reinforce the need to know exact instance names of each text box, and the path required to access each box. - The active components are movie clips with two states, active and inactive. The states are shown by the use of different graphics, on two frames. They should prevent few difficulties, providing that the text box can be accessed, as for the registers above. - The button components are straightforward to create. Explain to students how these buttons can be used to check a number of the other components which have just been produced, by inserting and clearing text into boxes, for example. - The bus clips are probably the most time consuming to create, but should not create too many difficulties providing students already know how to use tweening. (Use the classic tween option if using CS3 or above). The basic operation is the same for each bus: - When active, the bus clip is sent to frame 2. The red line is drawn on top of the motherboard bus from this frame, througout the clip. - A star graphic is tweened to follow the path of the bus, from one component to another. A separate tween is required for each “bend” in the bus. The number of frames used for each tween depends on the distance the star has to travel. - The actions layer holds stop() commands to prevent the clip from looping. - You may wish to demonstrate creating the first bus with the whole group, and then monitor how students create the other bus clips afterwards. - Use the next button (or prev button) to test each bus works by sending the clip to frame two, and playing the tween. As stated previously, the main problem in setting up the model lies in the number of components, and the need to check that instance names have been used correctly. Ask students to check each other’s work in pairs to ensure all components are working correctly before beginning to code the model. | 45 minutes |
| Activity: Write the code for the CPU model | Introduce the second stage in creating the model, which is to write the code. None of the code needed to control the model is particularly complex (there are no great logical problems to solve), but there are a lot of separate, small functions, which mostly make the components active (gotoAndPlay(2)) or change the text in a box. The one new piece of code is the switch…case statement, which is used to call each of the many (over 40) small functions to operate the model. Explain how this statement works, by referring to the appropriate slides in the demonstration (slide 9). Check students understand the similarity between switch…case and a number of separate IF statements. Provide an overview of the main tasks the code has to perform: - The Next button takes the user through the model one “clock tick” at a time, incrementing a variable called clickCount to do this. - Each function makes particular components active, or inactive. Refer to the Code Guide to find out what each function has to do. Several functions can be “recycled”, as they perform the same tasks on each program cycle. - The resetParts() function and the replay button function reset components to their inactive states. Similarly, provide an overview of what each program cycle does, and how these link to the main tasks of adding two numbers, and comparing the answer with the answer input by the user. Students should then be able to work through the demonstration independently, following these stages: - Set up the components to display their initial states and messages. This state is restored when the replay button is clicked, and the resetParts() function is called. Test that this code works. - Create the first “click” function. - Use the switch case statement to call the “click” functions, depending upon the value of the clickCount variable. - Display the user messages, in the appropriate sequence. Check students understand that the message often predicts what is going to happen next, so that the user can read the message, and then see it in operation. - Create further “click” functions from 2 through to 10. These are shown in the demonstration, so should provide little difficulty as they simply have to be followed according to the screen display. - Create the functions for the second program cycle. This is to be done independently, editing the functions for the first cycle as required. - Create the first six functions for the third cycle. Again, these can be copied from the first cycle and edited. The next three functions (27 to 29) add the two numbers, and are shown in the demonstration. - The fourth program cycle repeats 8 of the click functions. The next two functions (39, 40) are shown, as these store the result of the comparison in the Flags register. - The fifth program cycle repeats the fetch process for its first five functions. It then implements the JUMP IF instruction in the final three functions as the user’s answer was incorrect. These three functions are shown in the demonstration. - Use an IF statement in the next button function to prevent the clickCount variable incrementing beyond the scope of the click functions. While much of this code is not challenging, it does require concentration and thoroughness to make sure each function works as expected, and is edited correctly. Most importantly, following the sequence of functions required to operate the model strongly reinforces understanding of how the CPU works, and the notion that it is fundamentally a very “mechanical” process. It is this “mechanical” aspect that actually provides the difficult of coding the model. This is a useful point to discuss with students afterwards, especially for those who may not successfully complete the whole process in the time available. | At least one hour; probably longer |
| Activity: Challenge; produce a second CPU model | This challenge is intended as an extension task for more able students who may complete the first model ahead of others in the class. It checks whether a user has input a correct letter as part of a password (and so uses text input instead of numbers). It does not require the same number of functions, as there are only three program cycles, which follow the same process as in the first model. Challenge students to code the whole model from scratch, if time permits. | Up to one hour |
| Review Key points | Use the questions on the final slide in the tutorial to emphasise the key learning points in the tutorial. Choose questions according to how far the class has progressed during each lesson in the time available. | 10 minutes |
| Resources | - Computers 4 iDiots tutorial 5 - Zipped folder of resources (from the Resources link). Model graphics and the CPU Code Guide document. (It is useful to have printed copies of this document available). - Example models from the (Examples link). - You may wish to have some actual computer components available (motherboard, CPU, hard drive, RAM) to pass around the class to reinforce what they see on the screen. |  |